Skyline Aviation




   
               B200C Checklist 
V2.1  14 Apr 2012


NORMAL CHECKLIST

BEFORE START
Medical Oxygen……………………
……………....…………………………..ON

PAX Brief……………………………
…………..….……..Wx / ETI/ FN BRIEF

Pilot Seat Belts……..……………
………………….....………… …SECURED
Brakes………………………………
……………………………………...RESET

Power/Prop levers………………
…………………………………………...SET

Pressurisation
SET

Battery/GPU
CHECK ON & VOLTAGE
Ice Vanes
OPEN

Fuel Contents
CHECKED
Doors Annuciator
…..………………………………………………OFF
Aircraft Oxygen………………………
……………………….......................ON

Beacon
ON

ATS Start Clearance
OBTAINED

AFTER START

Fuel………………………………………………………………...…REVIEWED

T/O Speed Bugs
SET

Instruments
CHECKED & SET
Nav Aids/GPS
SET FOR DEPARTURE

Environmental
..AUTO & SET
Trims
3 SET

Go Around
ON

HDG Select
ON

ALT Select
ARMED

Medical Power
ON

GND Transport 
ORDERED
BEFORE TAXI
Cabin  FSB Sign……………………………………
…………………….….ON

Brakes
CHECKED

FLT Controls
FULL & FREE

Manual Feather
CHECKED

Flaps
SET 40?
Radar
STBY & UP

Auto Feather
ARMED

Prop Levers
FULL FORWARD
ATC Clearance
COPIED / ALERT SET

Comms for Departure
SET

T/O & SID Brief………………………………
………………….COMPLETED
Cabin Crew Report…………...……………
…………………………..READY
LINE UP
Transponder
ON

Annuciators
CHECKED

Auto Ignition 
ARMED

LDG/RECOG Lights
ON

Strobes
ON

Ice Protection (Pitot/Props/Vents/Stall/W-shield)
ON
Ice Vanes …………………………………………………..………………OPEN

Condition Levers
HIGH IDLE

AFTER TAKE OFF
Landing Gear
UP/LDG LIGHTS OFF

Flaps
UP

Yaw Damper
ENGAGED

Ice Vanes/Protection
AS REQUIRED

Pressurisation
DIFF/CABIN CHECKED

Power
SET

Engine Instruments
CHECKED

Auto Feather
OFF

TRANSITION ALTITUDE 13000FT
Altimeters
SET 1013 & X CHECKED

Alt Alerter
CLRD FL SET

Pressurisation
CHECKED

Recogs
OFF

FSB/Pax Comfort….………………………
………..….…….OFF/CHECKED

CRUISE 

Cruise Power
SET

Engine Instruments
MONITER

ATIS………..……………………………………………..………..……..COPIED

Arr/Appr/MA Brief………………..……………………….…..….COMPLETED

FLT & NAV Instruments
X CHECK & UPDATED
Grnd Transport………………………………………….…………..ORDERED

DESCENT
Descent Clearance
OBTAINED

Approach Brief & DH
BRIEFED & SET

Pressurisation
SET

Ice Vanes/Protection
AS REQUIRED

Pax Brief
FASTENED,SECURE,CLEAR
TRANSITION LEVEL FL150
Altimeters
QNH SET & X CHECKED
Alt Alerter
CLRD ALT SET

Pressurisation
CHECKED

Auto Feather
ARMED

Brakes
CHECKED

FLT & NAV Instruments
SET & CHECKED

Recogs……………………………
……………………………………..……ON

FSB Sign……..……………………
…………………………………..…..…ON

PRE-LANDING

Ice Vanes/Protection
AS REQUIRED

Yaw Damper
OFF

Landing Gear
DOWN LOCKED/LDG LIGHTS ON
Flaps
SET
Props
FULL FORWARD

AFTER LANDING RUNWAY VACATED

Strobes
OFF

Lights
AS REQUIRED

Auto Feather & Ignition
OFF

Ice Vanes……………………………………………………….……EXTENDED
Ice Protection (Pitot/Props/Vents/Stall/W-shield) …………….………OFF
Flaps
UP

Transponder
OFF

Radar………………………………………………………………..…STBY/OFF
Trims(3)………………………………………………………….. ….RESET T/0
SHUTDOWN
Park Brake
SET
Blowers/Environmental
AUTO/OFF

Medical Power………………………………………………….……………OFF

Inverter
OFF

Avionics
OFF

ITT……………………………………………
……………….…..…STABLISED
Condition Levers
IDLE CUT OFF

Gang Bar
OFF

Aircraft & Medical Oxygen………………………………..……...………..OFF
                                                 Effective 14 April 2012                                              2

