Skyline Aviation Limited                           IEX 072
                                   V 1.0 May 08 
AS350 NIGHT OPERATIONS COCKPIT CHECKLIST

PRE TAKE OFF
· Fuel – Sufficient, check & note
· Radios/Nav Aids – Selected
· GPS – Set
· RadAlt – Set 1000ft
· A/H’s – Erect & set
· HSI/DI – Align
· Heading Bug - Set heading
· Caution Lights – Out
· T’s & P’s – Normal
· Lights – Cabin, Landing, Nightsun, Night Scanner
· Intentions – Brief crew
· Flight Following – Contact 
· Passengers/Doors - Secure
· Inadvertent IMC – Brief & approach plates
· MSA - Check 
· Departure Time – Note
· NVG – On Goggles  
· Lift-off - Cleared
AFTER TAKEOFF
· Landing Lights – Off
· Nightsun/Night Scanner – On
· NVG’s – On
· WX/Visibility - Review 

· Climb - Safe cruise altitude
CRUISE
· Situational Awareness – maintain, CRM
· Scan – Traffic/NVG’s
· Route/Track - Revise
· MSA – Revise/1000ft AGL
· Radios/Nav Aids - Check
· T’s & P’s- Check
· AMP’s – Check
· Fuel – Check
· Weather – Review
· IIMC - Revise

· Flight Following – Update
APPROACH
· ATIS/QNH – Obtain & select
· Navaids- Select
· Approach – Intentions, brief
· Clearance - Obtain
· Destination Lighting - Activate
· Minimum Descent Altitude – decide/declare
· T’s & P’s - Check
· Ground Comms – Establish
· Landing/Search Lights – On
· Reconnaissance – Carry out

· Hazards – Check & Note
AFTER LANDING
· Lights – OFF
· Time – Note
· Flight Following – Update
· Fuel – Check/sufficient return
INADVERTENT IMC 

The 4 “C’s” Rule for IIMC
· Control the helicopter & revert to instrument flight immediately, note altitude, airspeed and heading.
· Climb to MSA or maintain altitude to safely clear surrounding terrain.
· Course - carryout a rate one turn onto the reciprocal heading and maintain instrument flight until VMC.
· Communicate – contact nearest ATS or FISCOM and declare a Mayday & advise intentions.







